Comments on: draft-ietf-sip-location-conveyance-07

Detailed review comments received from at least:

- Richard Barnes (rbarnes@bbn.com)

- Keith Drage (drage@alcatel-lucent.com)

- Rishu Jain (rishu.jain@aricent.com)

- Shida Schubert (shida@ntt-at.com)

- Henning Schulzrinne (hgs@cs.columbia.edu)

- Umesh Sharma (USharma@veraznet.com)

- Hannes Tschofenig (Hannes.Tschofenig@gmx.net)

- Joszef Varga (jozsef.varga@nsn.com)

Note Henning also sent a marked up copy to the authors – not directly reproduced here.

| No. | Location | Comment type | Commentor | Comment | Resolution |
| --- | --- | --- | --- | --- | --- |
| 1 | General | Technical | Hannes Tschofenig | I actually have two questions in this context: * Shouldn't the "SIP Location Conveyance" document define this term and use it in it's document? * Is it actually true that the LS will always have an LG attached to it? Typically, the LS will just talk to some network technology specific LGs anyway. Now, I would define the LIS as: " Location Information Server (LIS): A LIS is a Location Server located in the Access Provider's domain that is either co-located with a Location Generator (LG) or has access to one or multiple LGs. The terms "Location Server", "Access Provider" and "Location Generator" are taken from RFC 3693. " | James Polk: with my Conveyance author hat off = nooooo...! Why would this (a LIS) be defined in a SIP spec (that talks about SIP user agents and SIP servers)? A LIS (if this exists as "an entity") would be a SIP UA - and not a SIP server - and that is covered in the Conveyance doc by talking about SIP UAs. |
| 2 | General | Technical | Shida Schubert | 1. Dealing with multiple location information. As geolocation can be populated with various location information some provided by UA, some provided by intermediaries, it may be useful for there to be an ability where UA can express preference among location information it provides. Currently the spec doesn't say anything about how UAS/server selects the location to be parsed when there are multiple location information. Related to this, section with UAS behavior should elaborate on when 424 is to be returned. Is it when one of the many location information was problematic? Or is it when all of which UAS tried failed? Obviously there will be variation but some guidelines may be helpful. Also, desirable behavior of UAS/proxy handling message with multiple location information may be desirable. Now if we don't want to deal with multiple location information, it may be better to change the relevant text from MAY to NOT RECOMMENDED. |  |
| 3 | General | Technical | Henning Schulzrinne | - The use of multiple location values is under-defined. There are some clear cases where this is useful or potentially useful, and others where it isn't. The reader is only left with some vague warnings that this could be "confusing". That is rather confusing. |  |
| 4 | General | Technical | Rishu Jain | 2) Location by reference overhead: Look at the following scenario (P - is also a location recipient): LIS LIS ^ ¦ ^ ¦ SUBSCRIBE ¦ ¦ NOTIFY SUBSCRIBE ¦ ¦NOTIFY ¦ ¦ ¦ ¦ UA -------> P1 ------------------> P2 -----------> INIVITE INVITE INVITE In order to take action based on "retransmission-allowed" or not, P1 has to subscribe to the LIS (Location Information Server), obtain the location and then take the action. The same has to be done by all the intermediate proxies. So is this not a overhead? |  |
| 5 | General | Technical | Rishu Jain | 3) "retransmission-allowed" Even if "retransmission-allowed" value is set to "no", can the proxy remove the PIDF location object from the message? RFC 3261 does not allow any proxy to add or delete the message body. So what is the use of "retransmission-allowed" with value "yes" or "no"? |  |
| 6 | General | Technical | Rishu Jain | 4) What is the behavior in following case: Retransmission-allowed Routing-query-allowed Transmission for Query ---------------------- --------------------- ---------------------- "no" not present ????? |  |
| 7 | General | Technical | Rishu Jain | 7) <sips: username@server.com> or <sips:server.com/username> are these same? Is the second form a valid SIP URI? |  |
| 8 | General | Technical | Rishu Jain | 8) Why only PIDF format is selected. |  |
| 9 | General | Technical | Rishu Jain | 9) Assume the case that UAC has sent his identity in the first INVITE. Callee would have subscribed to that identity and would get the caller's location. Now if caller's identity is changed and caller sent a new RE-INVITE message. Should callee then stop subscription with first location and should subscribe to the new location? There is no behaviour with respect to this is mentioned in UAS scope? |  |
| 10 | General | Technical | Shida Schubert | 3. General problem with debugging the warning code and 424 response. > Because the location URI can be added by proxy, when warning code is sent, it would be useful to include the location URI (cid, sips, pres etc.) that triggered the error/warning. This would allow UA to see if location was added by intermediary, which location was malformed etc. |  |
| 11 | General | Editorial, major | Martin Thomson | Editorial, Major: *: I'd like to see the new 'routing-query-allowed' defined in schema and in one of the examples (preferably the unencrypted one, since the proxy is unlikely to be able to see the encrypted LO). (Incidentally, is this parameter an extension, or is the intent to update RFC 4119?) |  |
| 12 | General | Editorial | Henning Schulzrinne | - The authors should avoid the passive voice and convoluted sentence constructions that comprise whole paragraphs; they make the document rather tedious to read. Often, antecedents of pronouns are unclear. |  |
| 13 | General | Editorial | Henning Schulzrinne | - The text is repetitive, but differs in subtle nuances upon repetition, so it is unclear what is normative behavior. |  |
| 14 | General | Editorial | Henning Schulzrinne | - A colon cannot appear in the middle of a sentence. |  |
| 15 | General | Editorial | Henning Schulzrinne | - Throughout, it should be "header field", not "header". |  |
| 16 | General | Editorial | Henning Schulzrinne | - There are lots of unexplained uses of MAY vs. may. MAY is used for some future actions of standards-writers instead of implementors. |  |
| 17 | General | Editorial | Henning Schulzrinne | - In several instances, SHOULD or SHOULD NOT is used without any guidance as to when the stated rule can be violated or what bad thing would happen if the rule is violated. |  |
| 18 | General | Editorial | Henning Schulzrinne | - It is unclear whether "routing" refers to "retargeting" or something else. |  |
| 19 | General | Editorial | Henning Schulzrinne | - The use of by-value and by-reference is confusing. All elements in the header field are by reference, as they refer to a location elsewhere, even if in the same message. Where cid: is meant and where this requires special SIP element behavior, it should simply be called out as such. (For example, if a data URL were to be used later, it would be clearly "by value" and maybe the only true by-value, but much of the by-value discussion doesn't apply.) |  |
| 20 | General | Editorial | Henning Schulzrinne | - The document should, after first use, stick to UA (or UAC and UAS, as appropriate). It alternates randomly between spelling it out or using the abbreviation. |  |
| 21 | General | Technical | Henning Schulzrinne | - As a side note, I was roundly denounced as being out of touch with reality when I called for the use of multi-part MIME in another context in Prague (SDP capability negotiation), since everybody except me knew that nobody implements multi-part. Similarly, S/MIME seems to be essentially unimplemented, judging from the SIPit reports. Should we be honest enough to say that this means that really only L-by-R will be supported in practice? |  |
| 22 | General | Technical | Richard Barnes | 1. The draft says several times that proxies cannot insert LbyV because they can't modify bodies. That seems like a small reason to remove a potentially important use case, and it seems to contradict the fact that the ABNF in section 3.2 allows for the use of "data:" URIs. |  |
| 23 | General | Technical | Richard Barnes | 2. More generally, the looseness of the criteria for allowable URIs could also allow for URIs that convey LI outside of a PIDF-LO, either by reference or by value (e.g., the "geo:" URI defined in draft-mayrhofer-geo-uri). Perhaps this could be restricted to the usage "sip:", "sips:", and "pres:" defined by this document, plus any specific LbyR protocols later defined. |  |
| 24 | General | Technical | Richard Barnes | 3. Likewise, I don't see anything that says that the body part indicated by a "cid:" URI or returned by a reference MUST be a PIDF-LO (although this is said throughout). |  |
| 25 | General | Technical | Richard Barnes | 4. Several times, the draft says that it's only describing a "push", but there are at least two places it defines mechanisms for "pull": The inclusion of the "geolocation" option tag in a Require header, and the description of the usage of SIP as a dereferencing protocol. |  |
| 26 | General | Technical | Richard Barnes | 5. The routing-query-allowed element for PIDF-LO is never defined. This definition would be a better place for a lot of the discussion in section 5.3 (just in the introduction, not subordinate bullets). |  |
| 27 | General | Technical | Richard Barnes | 6. Several times, the document assumes that content protected with S/MIME is encrypted, when it could be signed, but otherwise readable. In most cases where use of S/MIME for encryption causes problems, use of S/MIME for signing only doesn't, and these cases should be noted. |  |
| 28 | General | Technical | Richard Barnes | 7. Several sections start out by talking about how sensitive location information is. This repetition should be removed. |  |
| 29 | General | Technical | Keith Drage | - General: usage of Geolocation and location bodies in responses. Obviously errors cannot be used in response to this, and location based routeing at a proxy is inappropriate. If we are allowing location in responses, we need to be clearer about which bit of the procedures apply, and which do not. This should probably be specifically worked in section 5. |  |
| 30 | General | Technical | Keith Drage | - Dialog Usage of new 424? RFC 4485 (http://www.ietf.org/rfc/rfc4485.txt) section 5 specifies: Dialog Usages: SIP allows for requests that normally create their own dialog (such as SUBSCRIBE) to be used within a dialog created by another method (such as INVITE). In such a case, there are said to be multiple usages of that dialog. Extensions SHOULD consider their interaction with dialog usages. In particular, extensions that define new error response codes SHOULD describe whether that response code causes the dialog and all usages to terminate, or just a specific usage. |  |
| 31 | Title | Editorial | Joszef Varga | Session Initiation Protocol Location Conveyance  Location Conveyance for the Session Initiation Protocol (SIP) |  |
| 31A | 1 | Editorial | Hannes Tschoenig | The introduction seems to have grown over time and at several paragraphs it now says "This document" or in "In this document" Maybe someone would like to ensure that it reads nicely rather than patched together. |  |
| 32 | 1 | Editorial | Joszef Varga | 1st paragraph This document describes how Location can be "conveyed" (that is, sent on the Internet) from a SIP user agent, or in some circumstances a proxy server acting on behalf of a user agent, to another entity using the SIP [RFC3261] protocol. Here "Location" is a description of the physical geographical area where a User Agent currently exists. This document uses the term "conveyance" to describe scenarios in which a SIP user agent client (UAC) is telling or informing a user agent server (UAS) where the UAC is. This is different from a UAC asking or seeking the location of where the UAS is. Conveyance is a push model, where seeking is a pull model, and therefore not discussed here. - should "conveyance" be "location conveyance" - delete "telling or" |  |
| 32A | 1 | Editorial | Hannes Tschoenig | " This document describes how Location can be "conveyed" (that is, sent on the Internet) from a SIP user agent, or in some circumstances a proxy server acting on behalf of a user agent, to another entity using the SIP [RFC3261] protocol. ^^^^^^^^^^^^^^^^^^^^^^^^^^ " Either write using SIP [..] or using the Session Initiation Protocol [...]. |  |
| 33 | 1 | NIT | Shida Schubert | 1. Section 1, 1nd paragraph, 3rd sentence. > The recipient of the location is not UAS alone as noted in the 2nd paragraph of this section, so you may want to say server or UAS & proxy rather than UAS alone. |  |
| 33A | 1 | Technical | Hannes Tschoenig | Change from: " This document describes how Location can be "conveyed" (that is, sent on the Internet) from a SIP user agent, or in some circumstances a proxy server acting on behalf of a user agent, to another entity using the SIP [RFC3261] protocol. Here "Location" is a description of the physical geographical area where a User Agent currently exists. This document uses the term "conveyance" to describe scenarios in which a SIP user agent client (UAC) is telling or informing a user agent server (UAS) where the UAC is. This is different from a UAC asking or seeking the location of where the UAS is. Conveyance is a push model, where seeking is a pull model, and therefore not discussed here. " to: " This document describes how geodetic and/or civic location information can be conveyed from a SIP user agent, or in some circumstances a proxy server acting on behalf of a user agent, to another entity using the SIP [RFC3261] protocol. This document does not describe mechanisms for subscription and notification. " Regarding the last sentence I wonder why you this document indicates that Geolocation header may appear in a SUBSCRIBE / NOTIFY message. |  |
| 33B | 1 | Technical | Hannes Tschoenig | Regarding: " This document uses the term "conveyance" to describe scenarios in which a SIP user agent client (UAC) is telling or informing a user agent server (UAS) where the UAC is. " I am not sure that this document is associating semantic to the location information with respect to the UAC. It just carries the location information from A to B. I believe that the information carried inside the PIDF (entity attribute) would tell to which entity this location information refers to. |  |
| 33C | 1 | Technical | Hannes Tschoenig | Regarding: "Conveyance is a push model, where seeking is a pull model, and therefore not discussed here. " I would omit this push vs. pull discussion since the draft also talks about LbyR and then these things get fuzzy. Please use the following terms as capitalized words since they refer to terms introduced in the GEOPRIV requirement RFC. We did this in other docs as well: target => Target, using protocol => Using Protocol, location server => Location Server, location object => Location Object, location recipient => Location Recipient |  |
| 34 | 1 | Technical | Umesh Sharma | >1. Introduction > >> It defines a "target" as the entity whose location is being >transmitted, in this case, it is > >>the user agent's (UA) location. >Target is probably only the "User Agent(UA)" (or the physical box in >which UA is present). I reworded the first part of the second paragraph to: "Geographic location in the IETF is discussed in RFC 3693 (Geopriv Requirements) [RFC3693]. It defines a "target" as the entity whose location is being sought. In this case, this is the user agent's (UA) location. " --- US: I am afraid it still seems to mean the same. This-->target-->UA's location I mean This-->target-->UA UA's Location would be represented as PID-LO. Otherwise in the reworded para: If you say something like "In this case, this is the user agent whose location is desired" etc. then UA becomes the entity. --- |  |
| 35 | 1 | Editorial | Joszef Varga | 2nd paragraph Geographic location in the IETF is discussed in RFC 3693 (Geopriv Requirements) [RFC3693]. It defines a "target" as the entity whose location is being transmitted, in this case, it is the user agent's (UA) location. A [RFC3693] "using protocol" defines how a "location server" transmits a "location object" to a "location recipient" while maintaining the contained privacy intentions of the target intact. This document describes the extension to SIP for how it complies with the using protocol requirements, where the location server is a User Agent or Proxy Server and the location recipient is another User Agent or Proxy Server. - Capitalisation consistent with 3693, 4119, e.g. "Target" (throughout document) - Capitalisation of User Agent and use of abbreviation – do search replace throughout document - LS can be the UA (by-value case) or a separate entity (by-reference) see the figure 1 in the requirement doc |  |
| 35A | General | Editorial | Hannes Tschoenig | The term 'Location' is written with a capital letter (see for example first paragraph in Section 1) whereas other RFC3693 terms that are meant to be written with a capital letter aren't (example: 'target', 'location object', 'using protocol', 'location recipient'). I suggest to stick with RFC3693. |  |
| 36 | 1 | NIT | Shida Schubert | 2. Section 1, 3rd paragraph, 2nd sentence. > I think you mean "target" where the text says thing. OLD: A PIDF-LO is an XML Schema specifically for carrying geographic location of a thing. NEW: A PIDF-LO is an XML Schema specifically for carrying geographic location of a target. |  |
| 37 | 1 | Editorial | Joszef Varga | (3rd para) Location can be transmitted by-value or by-reference. The "value" in this SIP extension is in the form of a Presence Information Data Format - Location Object, or PIDF-LO, as described in [RFC4119]. A PIDF-LO is an XML Schema specifically for carrying geographic location of a thing. Location-by-value refers to a user agent including a PIDF-LO as a body part of a SIP message, sending that location object to another SIP element. Location-by-reference refers to a user agent or proxy server including a URI in a SIP message which can be exchanged by a location recipient for a location object, in the form of a PIDF-LO. - Aganst final location recipient: What about Proxy Server? Location based routing in case location-by-reference used (e.g. e-call) |  |
| 38A | 1 | Editorial | Hannes Tschoenig | Regarding: " A PIDF-LO is an XML Schema specifically for carrying geographic location of a thing. " A PIDF-LO is an XML document extending PIDF to carry location information. |  |
| 38 | 1 | Editorial | Joszef Varga | (4th para) As recited in RFC 3693, location often must be kept private. The location object (PIDF-LO) contains rules which are binding on the location recipient and controls onward distribution and retention of the location. This document describes the security and privacy considerations that must be applied to location conveyed with SIP. - "location" OK without caps, I think the introduction of the term"Location" at the beginning is unnecessary |  |
| 38A | 1 | Editorial | Hannes Tschoenig | I suggest to rewrite the following paragraph from: " As recited in RFC3693, location often must be kept private. The location object (PIDF-LO) contains rules which are binding on the location recipient and controls onward distribution and retention of the location. This document describes the security and privacy considerations that must be applied to location conveyed with SIP. " to: " The Location Object contains rules which provide guidance for the Location Recipient on the onward distribution and retention of the received location information. This document describes the security and privacy considerations that must be applied to location conveyed with SIP. " |  |
| 39 | 1 | Technical | Umesh Sharma | 5th para. > >> ... SIP request, where the choice of the next hop (and usually, the > >> outgoing Request URI) is determined by the location of the UAC > >> which is in the message by-value or by-reference > >May consider rewording like >"... is determined by the location of the UAC which may be conveyed in >the message." >or ".. is determined by the location of the UAC which may be present as >by-vlaue or by-reference." >etc. Here, I am reading that you want the text to change from a case in which location is in the message, to one in which location *may* be in the message. While I agree completely that location can only be a MAY in general, for location based routing (the subject of this part of the text, we should indicate that location is present in order to perform this location based routing. Do you agree? --- US: agreed here(MAY part). Additionally, I wanted the sentence to be re-formatted for a better placing of the "by-value or by-reference" part, but that is non-issue. ---- |  |
| 40 | 1 | Editorial | Joszef Varga | (5th para) Often, location is sent from the User Agent Client to the User Agent Server, or vice versa for purposes that are beyond the scope of this document. Another use for location is location-based routing of a SIP request, where the choice of the next hop (and usually, the outgoing Request URI) is determined by the location of the UAC which is in the message by-value or by-reference. This document describes how location may be conveyed from the UAC, or a proxy acting on its behalf, to a routing proxy. How the location is actually used to determine the next hop or Request-URI is beyond the scope of this document. - Delete " Often, location is sent from the User Agent Client to the User Agent Server, or vice versa for purposes that are beyond the scope of this document"? - In relation to "SIP" is it protocol independent general text. |  |
| 41 | 1 | Editorial | Keith Drage | - 1, 5th para: Often, location is sent from the User Agent Client to the User Agent Server, or vice versa for purposes that are beyond the scope of this document. Another use for location is location-based routing of a SIP request, where the choice of the next hop (and usually, the outgoing Request URI) is determined by the location of the UAC which is in the message by-value or by-reference. This document describes how location may be conveyed from the UAC, or a proxy acting on its behalf, to a routing proxy. How the location is actually used to determine the next hop or Request-URI is beyond the scope of this document. change "may" to "can". |  |
| 41A | 1 | Editorial | Hannes Tschoenig | I don't know what this sentence should mean in Section 1 at this part of the section. " Often, location is sent from the User Agent Client to the User Agent Server, or vice versa for purposes that are beyond the scope of this document. " There seems to be a copy-and-paste problem. |  |
| 42 | 1 | Editorial | Richard Barnes | (6th para) Section 1, paragraph starting "The Geolocation header is introduced..." This paragraph should be broken into multiple sentences. |  |
| 43 | 1 | Editorial | Keith Drae | - 1, 6th para: The Geolocation header is introduced to signify that location is included in a SIP message to provide either a content identifier (cid:) pointer to the body part containing the UA's PIDF-LO, or a location-by-reference URI that may subsequently be "dereferenced" by a using protocol (which may be SIP or another protocol). change "may" to "can". |  |
| 43A | 1 | Editorial | Hannes Tschoenig | You write: " The Geolocation header is introduced to signify that location is included in a SIP message to provide either a content identifier (cid:) pointer to the body part containing the UAs PIDF-LO, or a location-by-reference URI that may subsequently be "dereferenced" by a using protocol (which may be SIP or another protocol). " Please delete the following part " by a using protocol (which may be SIP or another protocol)" since we haven't concluded in our discussion that the dereferencing protocol is indeed a using protocol. Actually, I think that the entire paragraph is not particularly useful in the introduction and the subsequent paragraph fits much better to the previous one. |  |
| 43B | 1 | Technical | Hannes Tschoenig | Emergency case: I believe that all occurrences of emergency services should be deleted from the document since this document is generic in its nature. Let the Phone BCP deal with the emergency issue since it has todo so anyway |  |
| 43C | 1 | Editorial | Hannes Tschoenig | Last para Put a comma before "which". Put a comma after "i.e." and "e.g." |  |
| 44 | 1 | Editorial | Joszef Varga | (last para) Capitalise "SIP" |  |
| 45 | 1 | Editorial | Keith Drage | - 1, last para: In this document, we frequently refer to the "emergency case". This refers to a specific, important use of sip location conveyance where the location of the caller is used to determine which Public Safety Answering Point (PSAP) should receive an emergency call request for help (e.g. a call to 1-1-2 or 9-1-1). This is an example of location-based routing. The location conveyed is also used by the PSAP to dispatch first responders to the caller's location. There are special security considerations which make the emergency case unique, compared to a normal location conveyance within SIP. change "should" to "is expected to" to avoid RFC 2119 type language here. |  |
| 46 | 2 | Editorial | Joszef Varga | "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" I guess these should be always all caps |  |
| 47 | 3.1 | NIT | Shida Schubert | 3. Section 3.1 paragraph 1, 2nd sentence > Missing a closing bracket. OLD: The Geolocation header contains either a URI which may be a "cid:" URI (Content Identification, per [RFC2392] NEW: The Geolocation header contains either a URI which may be a "cid:" URI (Content Identification, per [RFC2392]) |  |
| 48 | 3.1 | Editorial | Joszef Varga | This document creates a new SIP header: Geolocation. The Geolocation header contains either a URI which may be a "cid:" URI (Content Identification, per [RFC2392], or a location-by-reference URI to be dereferenced by a location recipient to retrieve the location of the UAC. - "creates"  "defines" - "header"  "header field" and throughout document - cid: Why not simply cid URI? As for sip, sips, pres. - UAC  Target UA |  |
| 49 | 3.1 | Editorial | Henning Schulzrinne | - Unless the draft is trying to win the "longest URI parameter" competition, something shorter such as "used-for-routing" would be sufficient. |  |
| 50 | 3.1 | Technical | Umesh Sharma | >3.1 > > >> Since body parts may not be inserted by a proxy server, >location-by-value > >> cannot be inserted by a proxy. >A UA's proxy or server would like to insert it by-value, where a >particular Location Server may not be globally accessible ? Here, I am not sure I understand your question. RFC 3261 does not allow SIP Proxies from inserting message bodies, so even if the proxy knows where the UAC is, it cannot insert this location information by-value. The terminology "Location Server" in Geopriv is the IP entity that builds the PIDF-LO and is first to transmit it. Since proxies cannot insert location by-value, proxies cannot be Location Servers. UACs can be location servers, and B2BUAs can be location servers. ---- US: That would be the Location Generators (entities that build PIDF-LO), Location Servers would receive these PIDF-LO from the LGs (RFC3693) and/or receive subscriptions from LRs. ---- Within all this, you mention the location server "may not be globally accessible". This tells me you are actually referring to the server where the location-by-reference URI points to (i.e. where the UAC's PIDF-LO is - waiting to be accessed - which is called dereferencing). This is not a "Location Server" in Geopriv, and RFC 3693 defines this. ----- US: But here I think this(the server) would correspond to an entity like Presence Agent that would respond to dereferencing SUBSCRIBE requests with NOTIFY's but this is one of the function of a Location Server [RFC2693]. ---- This document already states the dereference server needs to be accessible if location is conveyed by-reference, otherwise location is not conveyed (which is the point of this extension). ---- Ok ----- |  |
| 51 | 3.1 | Editorial | Richard Barnes | (1st para) Section 3.1, paragraph starting "This document creates..." Missing closing parenthesis after "[RFC2392]". |  |
| 52 | 3.1 | Editorial | Keith Drage | - 3.1, 1st para: This document creates a new SIP header: Geolocation. The Geolocation header contains either a URI which may be a "cid:" URI (Content Identification, per [RFC2392], or a location-by-reference URI to be dereferenced by a location recipient to retrieve the location of the UAC. Change "may" to "can". |  |
| 53 | 3.1 | Editorial | Joszef Varga | (3rd para) If the URI in the Geolocation header is a scheme other than "cid:", another protocol operation is needed by the message recipient to obtain the location of the target (UA). This is location-by-reference. This document describes how a SIP presence subscription [RFC3856] can be used as a dereference protocol. - 1st sentence  " If the Geolocation header field value is a in the URI scheme other than "cid:"," - "message"  "SIP message" |  |
| 54 | 3.1 | Editorial | Josezef Varga | (4th para) The Geolocation header, either with the PIDF-LO in a body or as a location-by-reference URI, may be included by a User Agent in a message. A proxy server may assert location of the UA by inserting the header, which must specify a location-by-reference URI. Since body parts may not be inserted by a proxy server, location-by-value cannot be inserted by a proxy. - message  SIP message - proxy  Proxy Server |  |
| 55 | 3.1 | Editorial | Keith Drage | - 3.1, 4th para: The Geolocation header, either with the PIDF-LO in a body or as a location-by-reference URI, may be included by a User Agent in a message. A proxy server may assert location of the UA by inserting the header, which must specify a location-by-reference URI. Since body parts may not be inserted by a proxy server, location-by-value cannot be inserted by a proxy. Change "may" to "can". |  |
| 56 | 3.1 | Technical | Umesh Sharma | 5th para > >> The "inserted-by" parameter has values of "endpoint" or "server", > >> indicative of which entry ... >indicative of which entity are you taking issue with the word "indicative"? If so, how about me changing this to "... indicating which entity..." ---------- US: ok: "...indicating which entity added location to the message." ------------ |  |
| 57 | 3.1 | Editorial | Keith Drage | - 3.1, 5th para: The Geolocation header may have parameters that are associated with a URI in the header. The "inserted-by" parameter has values of "endpoint" or "server", indicative of which entry added location to the message. This header parameter MAY be added every time a new location is added into a message. Change first "may" to "can". For the final sentence, I would prefer this appeared elsewhere in the document as I would like to keep RFC 2119 language out of a section entitied "overview". |  |
| 58 | 3.1 | Technical | Umesh Sharma | > >>If a SIP message is routed within the network based on the location > >>contained within that message, the "message-routed-on-this-uri".. > >"within the network" -- the scope of network is not clear. fair -- how about I take "within the network" out of the sentence? --- ok. --- |  |
| 59 | 3.1 | Technical | Richard Barnes | Section 3.1, paragraph starting "If a SIP message is routed..." The requirement that header parameters SHOULD NOT be deleted seems like it should be a MUST NOT. What's the case where a parameter SHOULD be deleted? |  |
| 60 | 3.1 | Editorial | Joszef Varga | (6th para) If a SIP message is routed within the network based on the location contained within that message, the "message-routed-on-this-uri" parameter MUST be added as a header parameter of the URI used to route the message. Once a header parameter is added to a Geolocation header, it SHOULD NOT be deleted in transit to the ultimate destination. - "contained"  "contained or referenced". - Add " in the Geolocation header field" to " header parameter of the URI" - with ref to "SHOULD NOT be deleted" What happens if afterwards a proxy has location info, adds another Geolocation header, and routes based on that? |  |
| 60A | 3.1 | Technical | Hannes Tschoenig | "message-routed-on-this-uri": This functionality was introduced quite late in the process. If I recall it correctly then it would be used for debugging purposes. Is this correct? (nothing more - right?) |  |
| 61 | 3.1 | Editorial | Joszef Varga | 8th para This document describes an extension to PIDF-LO, the "routing-query-allowed" element, defined in the 'usage-rules' element. When set, this allows an element receiving location to transmit the location to another element to obtain routing information. When used in conjunction with the "retransmission-allowed" element, the rule maker can control distribution of the location information for location-based routing. - "routing-query-allowed" element: Add to IANA registration |  |
| 61A | 3.1 | Technical | Hannes Tschoenig | "retransmission-allowed": With the "retransmission-allowed" element, the rule maker can control distribution of the location information for location-based routing. It is clear how these rules are set with location information being added by an end point. In that particular case it is implicit that the end point want to use location information for routing. If it would not want to use location information for routing it would have to encrypt it anyway. Hence, is this functionality really necessary? |  |
| 62 | 3.2 | Technical | Rishu Jain | 6) Section 3.2: Use of the header in BYE, INFO and REFER Methods are allowed, although no purpose is known. Conveying location in a CANCEL, BYE, ACK or PRACK is not defined. Conveying location in "BYE" is defined or not? |  |
| 63 | 3.2 | NIT | Shida Schubert | 4. Section 3.2, paragraph 1, 1st sentence > It reads like the spec creates a new IANA registry.. IANA consideration covers all of which has to do with IANA, so I think all you have to say here is that the spec defines a new SIP header: Geolocation. |  |
| 64 | 3.2 | NIT | Shida Schubert | 5. Section 3.2, paragraph 1, 2nd sentence > The Geolocation heade MUST contain one of two types of URIs is misleading I believe what you want to say is "at least one of two types of URIs" OLD: The Geolocation header contains either a URI which may be a "cid:" URI (Content Identification, per [RFC2392] NEW: The Geolocation header contains either a URI which may be a "cid:" URI (Content Identification, per [RFC2392] |  |
| 64A | 3.2 | Editorial | Hannes Tschoenig | You write: " The Geolocation header MUST contain one two types of URIs: ^^^^^^^^^^^^ one of two types " |  |
| 65 | 3.2 | Editorial | Joszef Varga | (3rd para) A location-by-value content-ID (cid-url) [RFC2392] indicates which message body part contains location for this UA. - URL? |  |
| 66 | 3.2 | Technical | Richard Barnes | Section 3.2, paragraph with ABNF Since the Geolocation header can hold multiple values, should messages be forbidden to have multiple Geolocation headers? Also, this specification is very permissive about what URIs may be inserted. Perhaps it should be restricted to SIP, SIPS, PRES, and CID for now, with a provision for adding more in the future. |  |
| 67 | 3.2 | Technical | Richard Barnes | Section 3.2, paragraph starting "The cid-url is defined..." The meaning of the sentence "This URI MUST be present if location is by-value in a message" needs to be re-worded or deleted. As I read it, it would imply that if a message has a PIDF-LO body part (for any reason), and a Geolocation header, than that header must have a CID URI pointing to that body part. |  |
| 68 | 3.2 | NIT | Shida Schubert | 6. Section3.2, paragraph before the table, last sentence. > "is" is missing. OLD: Discussing location using the PUBLISH Request Method out of scope for this document. NEW: Discussing location using the PUBLISH Request Method is out of scope for this document. |  |
| 69 | 3.2 | Technical | Richard Barnes | Section 3.2, paragraph starting "Use of the header..." Use of the header in BYE is stated to be both allowed and undefined. |  |
| 70 | 3.2 | Technical | Umesh Sharma | >3.2 > >>contain one of two types of URIs:contain of the two types of URIs. > >>Discussing location using the PUBLISH Request Method out of scope > >>for >this document >]is[ out of scope for this document thanks, changed |  |
| 71 | 3.2 | NIT | Shida Schubert | 7. Section 3.2, 2nd last paragraph, 2nd sentence. > Confusing, needs to clarify that proxy is not allowed to modify the pre-existing URI values and associated parameters. OLD: A proxy MAY add the Geolocation header, but MUST NOT modify the contents of an existing Geolocation header. NEW: A proxy MAY add the Geolocation header, but MUST NOT modify any pre-existing URI values and its associated parameters of an existing Geolocation header. |  |
| 72 | 3.2 | NIT | Shida Schubert | 8. Section 3.2, 2nd last paragraph, last sentence. > location-by-reference URI not location-by-reference. OLD: Therefore, a Geolocation header added by a proxy MUST specify location-by-reference. NEW: Therefore, a Geolocation header added by a proxy MUST specify location-by-reference URI. |  |
| 73 | 3.2 | Technical | Henning Schulzrinne | - INFO and REFER are excluded. Why? (For example, INFO might be a good candidate for mid-call location updates; clearly, UPDATE is not unless you are also changing the session description.) |  |
| 74 | 3.2 | Technical | Henning Schulzrinne | - PUBLISH is excluded with some ominous-sounding warnings that have the flavor of "unspeakable horrors will happen to your children and grand children, but we dare not speak their name". Why not simply say that you could simply PUBLISH PIDF-LO as a described in the presence-for-location-info RFC? |  |
| 75 | 3.2 | Technical | Richard Barnes | Section 3.2, paragraph starting "The Geolocation header MAY be..." A Geolocation header added by a proxy could specify LbyV, e.g., if it contained a "data:" URI. |  |
| 75A | 3.2 | Technical | Hannes Tschoenig | * Privacy Rules You write: " Entities receiving location information MUST honor the usage element rules per RFC4119. Such entities MUST NOT alter the rule set. " The last sentence is not correct. Let me give you an example: A SIP UAC obtains location information from GPS and uses a PUBLISH to upload the PIDF-LO to the Location Server/ Presense Server. This server has rules that control access to location information. As part of these rules the content of the Location Object including the rules might be modified. |  |
| 76 | 3.3 | Editorial | Richard Barnes | Section 3.3, paragraph starting "The UAC can use whatever means..." Need an article before "SIP Intermediary". |  |
| 77 | 3.4 | NIT | Martin Thomson | Editorial, Minor: 3.4 & 9.4: Warnings refer to unsupported "schema". This should be "scheme" [RFC3986]. |  |
| 78 | 3.4 | Technical | Henning Schulzrinne | - The Warning header has no way to indicate which of several locations it refers to, thus making it useless in the common case that you have both a protected (high-res) location in the body and an unprotected non-cid reference. (See Marc's recent proposals for an example.) At best, one can guess by circumstances. |  |
| 79 | 3.4 | Technical | Henning Schulzrinne | - It is unclear whether the Warnings can appear only in a 424. |  |
| 80 | 3.4 | Technical | Henning Schulzrinne | (see also comment against 3.4.7 to put this in context) This, in general, illustrates that the plethora of Warnings is unlikely to be helpful. Almost all the conditions need to be fixed by humans, rather than by machine, and need significant extra information to be useful, presumably in some free-text format, such as the 424 body. If that information needs to be provided anyway, why bother with the hyper-specific warnings? After all, it's not like the UAC software is going to be saying "hmm, I really didn't want to include my cubicle number, but since the UAS insists, I guess I'll be nice and include it this time. But, hey, maybe the UAS is happy with the floor number?". |  |
| 81 | 3.4 | Editorial | Keith Drage | - 3.4, 3rd para: The Warning header allows for multiple warning codes be returned in the same response. If a location-by-reference is sent and the supplied scheme is not desired or cannot be processed, but more than one other scheme can be, the 424 response can list more than one code from the 720-724 range in the response. The UAC may subsequently retry the operation with one of the schemes supported or desired by the recipient. Change "may" to "can". |  |
| 82 | 3.4 | Editorial | Richard Barnes | (4th para) Section 3.4, paragraph starting "To illustrate this..." Replace "would be in here" with "is" |  |
| 83 | 3.4.1 | Technical | Richard Barnes | Section 3.4.1, paragraph starting "A Warning header ..." You refer to data-URLs here; it would be useful to have an Informative reference to RFC 2397. |  |
| 84 | 3.4.1 | Editorial | Henning Shulzrinne | - The 701 code should indicate which other warnings are subsumed by it. What value does it add compared to not including it? |  |
| 85 | 3.4.1 | Technical | Joszef Varga | (1st para) A Warning header with the code 701 "Location Format not supported" means the location format supplied in the request, by-value or by-reference, was not supported. This cause means the recipient understood that location was included in the message, but the format is not supported. Perhaps the format was a freeform text format or data-URL and the recipient only understood location in RFC 4119 PIDF-LO format (civic or coordinate). If a more specific Warning code is available, it MUST be used. For example, if the format is understood, but not desired, a 702 or 703 Warning header should be returned in a 424 response, depending on which location format is desired. The same applies to a location recipient that only understands one format and did not receive that format. For example, if a message containing coordinate formatted location arrives but the recipient only can process civic formatted location, a 703 Warning header should be returned in a 424 response. - Query as to whether really needed - should be  "SHOULD be" |  |
| 86 | 3.4.1 | Editorial | Keith Drage | - 3.4.1, 1st para: A Warning header with the code 701 "Location Format not supported" means the location format supplied in the request, by-value or by-reference, was not supported. This cause means the recipient understood that location was included in the message, but the format is not supported. Perhaps the format was a freeform text format or data-URL and the recipient only understood location in RFC 4119 PIDF-LO format (civic or coordinate). If a more specific Warning code is available, it MUST be used. For example, if the format is understood, but not desired, a 702 or 703 Warning header should be returned in a 424 response, depending on which location format is desired. The same applies to a location recipient that only understands one format and did not receive that format. For example, if a message containing coordinate formatted location arrives but the recipient only can process civic formatted location, a 703 Warning header should be returned in a 424 response. Query RFC 2119 language on use of word "should". Is it or isn't it. If it is not, change the wording to make clearer this is part of the example. If it is not, then make clear that it is a normative requirement by deleting "For example". |  |
| 87 | 3.4.1 | Editorial | Joszef Varga | (2nd para) Recommended warn-text: Location format not supported - f or F |  |
| 88 | 3.4.1 | Editorial | Keith Drage | - 3.4.1, 4th para: A proxy server may assert location of the UA by inserting the header, which must specify a location-by-reference URI. Since body parts may not be inserted by a proxy server, location-by-value cannot be inserted by a proxy. "must" or "MUST". If the requirements are elsewhere use a different word to "must". change "may not" to "cannot". |  |
| 89 | 3.4.2 | Editorial | Joszef Varga | (Title) Delete "Instead" |  |
| 90 | 3.4.3 | Editorial | Joszef Varga | (Title) Delete "Instead" |  |
| 91 | 3.4.5 | NIT | Shida Schubert | 9. Section 3.4.5 > I believe this warning code is appropriate when PIDF fetched via SUB/NOT doesn't contain the LO portion. |  |
| 92 | 3.4.5 | Technical | Henning Schulzrinne | - I don't know that the difference is between not being able to locate a URL and not finding the corresponding cid. |  |
| 93 | 3.4.5 | Editorial | Keith Drage | - 3.4.5, 1st para: A Warning header with the code 705 "Cannot find location" means location should have been in the message received, but the recipient cannot find it, either because it is not in the message, or it is encrypted/opaque to the recipient. "should have been" --> "was expected". |  |
| 94 | 3.4.6 | Technical | Richard Barnes | Security / Privacy Concerns: 6. The draft's stance on multiple locations seems to take an absolutist stance on multiple locations. In particular, Section 3.4.6 allows for multiple locations to cause a call to fail (which seems dangerous to me). This seems dangerous in an emergency context. |  |
| 95 | 3.4.6 | Technical | Shida Schubert | 2. Section 3.4.6 Warning code 706 Conflicting Locations Supplied > I understand its use, but considering proxy may add Geolocation header with reference URI, it's out of UA's control to restrict the number of location to be conveyed unless... 1. UA can tell the proxy not to add location 2. UA can find out if location was added by other entity in which case it can omit adding location itself and have the proxy add location or route a call through other proxy which may not add location. |  |
| 96 | 3.4.6 | Technical | Richard Barnes | Section 3.4.6 Allowing this response code would seem to encourage UASs to reject calls with multiple locations. This is obviously dangerous in an emergency setting. Also, it seems odd to include this in a 424 message that goes back to the UAC, since there may not be anything he can do about it (e.g., if a proxy is adding conflicting location information). |  |
| 97 | 3.4.6 | Technical | Joszef Varga | (2nd para) A typical reaction to receiving this warning code is to reduce the number of different locations supplied in the request, and send another message to the location recipient. - What happens if a proxy adds a conflicting location every time? Any priority / processing rules at the UAS? |  |
| 98 | 3.4.6 | Editorial | Joszef Varga | (last para) Warning: 706 alice.example.com "conflicting locations supplied" - Caps (on Warning name) |  |
| 99 | 3.4.7 | Technical | Henning Schulzrinne | - The 707 Warning indicates that the location is "incomplete". The value of this is unclear. Does it mean that the geo information has an insufficient number of digits? Does the receiver want cubicle-level information? Should it add digits until the warning disappears? Or does this refer to some syntactic deficiency or a logical deficiency, such as a missing 'country' field in PIDF-LO when a state is provided? |  |
| 100 | 3.4.7 | Editorial | Joszef Varga | (3rd para) Recommended warn-text: Conflicting Locations Supplied - L or l? |  |
| 101 | 3.4.7 | Editorial | Joszef Varga | (5th para) Warning: 706 alice.example.com "conflicting locations supplied" - L or l? |  |
| 101A | 3.4.9 | Technical | Hannes Tschoenig | This document talks about a LIS but does not define what it is :-) |  |
| 102 | 3.4.12 | Editorial | Keith Drage | - 3.4.12: There are two sections labelled such. |  |
| 103 | 3.4.12 | Editorial | Joszef Varga | (Title) 3.4.12 Warning code 720 Unsupported Schema - sip desired - "sip"  "SIP" ? |  |
| 104 | 3.4.12 | Technical, minor | Martin Thomson | Technical, Minor: 3.4.12: Note that providing a reference of this nature (which would allow for a dereference operation that does not use TLS) could constitute a potential privacy leak. UAs should only respect this request in an emergency. |  |
| 105 | 3.4.12 | Technical | Henning Schulzrinne | - The use of a Warning header to indicate desired URL schemes is inappropriate and has no precedent. If desired, an Accept-Geolocation header field would be more useful. You don't want to extend the Warning list each time a URI scheme is added. |  |
| 106 | 3.4.12 | Editorial | Joszef Varga | (3rd para) Recommended warn-text: unsupported schema - Add "- SIP desired" |  |
| 107 | 3.4.12A | Technical | Henning Schulzrinne | - The use of a Warning header to indicate desired URL schemes is inappropriate and has no precedent. If desired, an Accept-Geolocation header field would be more useful. You don't want to extend the Warning list each time a URI scheme is added. |  |
| 108 | 3.4.12A | Editorial | Joszef Varga | (Title) - Warning code 721 Unsupported Schema - sips desired - "sips"  "SIPS". |  |
| 109 | 3.4.12A | Editorial | Joszef Varga | (2nd para) Recommended warn-text: unsupported schema - Add "SIPS desired" |  |
| 110 | 3.4.13 | Technical | Henning Schulzrinne | - The use of a Warning header to indicate desired URL schemes is inappropriate and has no precedent. If desired, an Accept-Geolocation header field would be more useful. You don't want to extend the Warning list each time a URI scheme is added. |  |
| 111 | 3.4.13 | Editorial | Joszef Varga | (2nd para) Recommended warn-text: unsupported schema - Add "pres desired" |  |
| 112 | 3.5 | Technical | Umesh Sharma | >3.5 > >> Appearance of the option tag in the Require header is a request > >> for >location to > >> be conveyed. > From rfc3261 it is inferred that the UAS needs to understand the >conveyance information coming from UAC/etc. >Maybe an emergency call where there are two PSAPs where call could be >forwarded, the one who understands the Geo-LocationInfo ulitmately gets >the call. I think this is outside the scope of this doc. This topic should be asked in ECRIT, as they have 2 docs defining the devices BCP and the Emergency calling Framework. Hopefully there will be an emergency services network in front of the IP enabled PSAP to route the call to a PSAP that can understand this extension. That said, I believe what will happen is most (if not all?) IP based PSAPs understand this extension from the beginning. NENA and EMTEL are telling all PSAPs in North America and the EU to do this. Several other areas of the world are being told to do this also. Of course we cannot be assured every IP enabled PSAP in the world will, but it's likely to be based on something from the IETF, the ITU-T, 3GPP or one of the other SDOs that attended last year's Emergency SDO conference that agreed to this general mechanism to convey location to a PSAP from a SIP phone. --- US: Ok, but is this contradicting the statement in Section 5.2 which says "...A Require header with the geolocation option tag indicates the UAC is requiring the UAS understand this extension or else send an error response." as opposed to 3.5 "...Appearance of the option tag in the Require header is a request for location to be conveyed." --- |  |
| 113 | 3.5 | Technical | Shida Schubert | 5. Section 3.5, 2nd paragraph, 2nd sentence. > Semantics of Require header with "geolocation" option tag is unlear. > Section 3.5 reads as if it is to request UAS to send back location but Section 5.2 merely implies that UAS understands the specification. I believe there needs to be consistency with the semantic. |  |
| 114 | 3.5 | Technical | Richard Barnes | (3rd Para) Section 3.5, paragraph starting "A UAC SHOULD NOT include..." Need to define what the semantic would be when included. Is it requesting that a proxy insert a Geolocation header? |  |
| 115 | 3.6 | Technical | Joszef Varga | New clause to define the new value for the rule element? |  |
| 116 | 3.6 | NIT | Shida Schubert | 10. Section 3.6, 1st paragraph, 1st sentence. > I think what you are trying to say is taht, when Geolocation header is used to include a location-by-reference URI, sip, sips or pres URI SHOULD be used. Isnt' it? Right now, it reads like when Geolocation header is set, URI with one of the scheme should be set |  |
| 116A | 3.6 | Question | Hannes Tschoenig | Questions: You write: " A dereference of a location-by-reference URI using SUBSCRIBE is not violating a PIDF-LO 'retransmission-allowed' element value set to 'no', as the NOTIFY is the only message in this multi-message series of transactions that contains the UAC's location, with the location recipient being the only SIP element to receive location - which the purpose of this extension: to convey location to a specific destination. " I wasn't quite sure what you are about to say with this statement. If the Location Recipient receives a reference and it resolves it then you are saying that this is OK even if the retransmission-allowed flag says "no". Correct? |  |
| 116B | 3.6 | Technical | Hannes Tschoenig | * Using Protocol You write: " Other protocols used in the Location URI MUST be reviewed against the RFC3693 criteria for a using protocol. " Later, in Section 3.6 you write: " Use of other protocols for dereferencing of a pres: URI is not defined, and such use is subject to review against RFC3693 using protocol criteria. " I disagree that the protocol used to resolve the reference has to be a using protocol. Hence, I suggest to delete this sentence. |  |
| 117 | 4 | Editorial | Henning Schulzrinne | - The term "coordinates" is used to refer to geo-location, but not defined. This term also seems at odds with usage elsewhere. GEOPRIV should agree on one term for non-civic coordinates. |  |
| 117A | 4 | Technical | Hannes Tschoenig | * The examples pick unfortunate references (sips:server5.atlanta.example.com/alice123) that give me the impression that the reference contains the user identity. The document clearly needs to point out that this is not the case. The document also does not indicate that the reference is only kept for a certain period of time (soft-state). The document does not even mention security aspects when retrieving the reference. |  |
| 117B | 4 | Editorial | Hannes Tschoenig | The example contains a company name (<gp:provided-by>www.cisco.com</gp:provided-by>) although it should, according to the guidelines something like "www.example.com". |  |
| 118 | 4.1 | Editorial | Umesh Sharma | >4.1 > >>...indicates the content-ID location [RFC2392] within the multipart > >> message body of were location information is > >message body of ]where[ location information is Got it, thanks! |  |
| 119 | 4.1 | Technical, minor | Martin Thomson | Technical, Minor: 4.1 & 4.2: The example PIDF-LO documents have a few errors. Refer to draft-ietf-geopriv-pdif-lo-profile. Continued... |  |
| 120 | 4.1 | Technical, minor | Martin Thomson | 4.1 & 4.2: Remove the xmlns:gs attribute, it isn't used. |  |
| 121 | 4.1 | Technical, minor | Martin Thomson | 4.1: This Point format is deprecated (for a number of reasons). Recommend the following form: <gp:location-info> <gml:Point srsName="urn:ogc:def:crs:EPSG::4326"> <gml:pos>33.001111 -96.68142</gml:pos> </gml:Point> </gp:location-info> |  |
| 122 | 4.1 | Technical | Martin Thomson | 4.1 & 4.2: The "method" and "provided-by" elements are in the wrong context. An unfortunate consequence of the structure of the RFC 4119 schema is that the "usage-rules" will accept any old junk provided that it is in the "...:geopriv10" namespace. Therefore, the given documents validate, even if they aren't correct. Most likely those elements would be ignored by some UASs. Place the method and provided-by elements _after_ the usage-rules. |  |
| 123 | 4.1 | Technical, minor | Martin Thomson | 4.1 & 4.2: The content model for the "provided-by" element does not permit text. It only permits element content. <gp:provided-by>www.cisco.com</gp:provided-by> is not valid. |  |
| 124 | 4.2 | Editorial | Henning Schulzrinne | - The second example adds no value, just inflates the spec size. |  |
| 125 | 4.2 | Technical, minor | Martin Thomson | Technical, Minor: 4.1 & 4.2: The example PIDF-LO documents have a few errors. Refer to draft-ietf-geopriv-pdif-lo-profile. Continued... |  |
| 126 | 4.2 | Technical, minor | Martin Thomson | 4.1 & 4.2: Remove the xmlns:gs attribute, it isn't used. |  |
| 127 | 4.2 | Technical | Martin Thomson | 4.1 & 4.2: The "method" and "provided-by" elements are in the wrong context. An unfortunate consequence of the structure of the RFC 4119 schema is that the "usage-rules" will accept any old junk provided that it is in the "...:geopriv10" namespace. Therefore, the given documents validate, even if they aren't correct. Most likely those elements would be ignored by some UASs. Place the method and provided-by elements _after_ the usage-rules. |  |
| 128 | 4.2 | Technical, minor | Martin Thomson | 4.1 & 4.2: The content model for the "provided-by" element does not permit text. It only permits element content. <gp:provided-by>www.cisco.com</gp:provided-by> is not valid. |  |
| 129 | 4.2 | Technical, minor | Martin Thomson | 4.2: The "civilAddress" element should be replaced by a "civicAddress" (note the 'l' is replaced by 'c'). |  |
| 129A | 4.3 | Editorial | Hannes Tschoenig | Section 4.3 uses the term 'LIS' and does not mention what it is. Just avoid it. |  |
| 129B | 4.3 | Technical | Hannes Tschoenig | Delete this statement: " This will likely not suit some services already being considered in the IETF at the time of this writing, such as emergency calling. " |  |
| 129C | 5 | Technical | Hannes Tschoenig | * Security in Section 5 SIP Element Behavior I am confused with the text in Section 5. What MUST and what SHOULD be used when protecting a location-by-value and a location-by-reference? SHOULD for TLS usage with location-by-value and location-by-reference. No statement about usage of S/MIME. I wonder why self-signed certificates shouldn't be used as stated with: " Self-signed certificates SHOULD NOT be used for protecting PIDF, as the sender does not have a secure identity of the recipient. " For encryption (and that's what we are talking about here) it heavily depends on the way how the self-signed certificate was obtained. I might share my certificate with you and hence that's an extemely useful way todo compared to the usage of TLS in a hop-by-hop way where a number of entities see my location along the path. Identity info in the PIDF. In previous discussion I was wondering and asking the Geopriv group whether there is some semantic associated with the identity in the PIDF. If there is, then you cannot put anything you like into the PIDF. If there isn't then why should we ever want to put something meaningful in there since it greatly impacts the privacy risks. I see a problem with retargeting and forking where the Location Recipient is actually not know ahead of time and S/MIME usage is actually not really feasible. Would some guidance be useful? |  |
| 129D | 5 | Technical | Hannes Tschoenig | Section 5: " Possession of a dereferenceable location URI may be equivalent to possession of the location information itself and thus TLS SHOULD be used when sending location-by-reference. " What does this mean? You always have to use TLS when you do not use S/MIME regardless whether you send it by value or by reference. Exception: Emergency Services |  |
| 130 | 5 | NIT | Shida Schubert | 11. SEction 5, 1st paragraph, 3rd sentence. > term opposite UA sounds a bit odd. OLD: SIP endpoints SHOULD implement S/MIME to encrypt the PIDF-LO message body (part) end-to-end when the intended recipient is the opposite UA. NEW: SIP endpoints SHOULD implement S/MIME to encrypt the PIDF-LO message body (part) end-to-end when the intended recipient is another UA. |  |
| 131 | 5 | NIT | Shida Schubert | 12. Section 5, 1st paragraph, last sentence > Unclear whether the target of TLS is the URI contained in the Geolocation header or the signalling path which message holding the header is conveyed. |  |
| 132 | 5 | Technical | Richard Barnes | (1st para) Section 5, paragraph starting "Because a person's location..." Location in general is considered sensitive (not just people), and the protocol is designed to describe more than people. |  |
| 133 | 5 | Editorial | Keith Drage | - 5, 1st para: Because a person's location is generally considered to be sensitive in nature, privacy of the location information must be protected when transmitting such information. Section 26 of [RFC3261] defines the security functionality SIPS for transporting SIP messages with either TLS or IPSec, and S/MIME for encrypting message bodies from SIP intermediaries that would otherwise have access to reading the clear-text bodies. SIP endpoints SHOULD implement S/MIME to encrypt the PIDF-LO message body (part) end-to-end when the intended recipient is the opposite UA. The SIPS-URI from [RFC3261] MUST be implemented for message protection (message integrity and confidentiality) and SHOULD be used when S/MIME is not used. Possession of a dereferenceable location URI may be equivalent to possession of the location information itself and thus TLS SHOULD be used when sending location-by-reference. Change "may" to "can". |  |
| 134 | 5 | NIT | Shida Schubert | 13. Section 5, 2nd paragraph, last sentence. > "does" or "to" is missing. OLD: When using location-by-reference, it is RECOMMENDED that the URI not contain any identifying information (for example use 3fg5t5yqw@example.com rather than alice@example.com) NEW: When using location-by-reference, it is RECOMMENDED that the URI (does¦to) not contain any identifying information (for example use 3fg5t5yqw@example.com rather than alice@example.com) |  |
| 135 | 5 | Technical | Richard Barnes | (2nd para) Section 5, paragraph starting "A PIDF includes identity information..." I don't really see the value of anonymizing the location in this case, since it's already contained in a SIP message that has identifying information. |  |
| 136 | 5 | Editorial | Joszef Varga | (3rd para) The entities receiving location MUST obey the privacy and security rules in the PIDF-LO as described in RFC 4119, regarding retransmission and retention. - 1st line looks too short |  |
| 137 | 5 | Technical | Richard Barnes | (4th para) Section 5, paragraph starting "Self-signed certificates..." This paragraph is out of scope. It's general PIDF/GEOPRIV concern. |  |
| 138 | 5 | Technical | Keith Drage | - 5, last 2 para: More than one location representation or format, for example: civic and coordinate, MAY be included in the same message body part, but all MUST point at the same position on the earth. Multiple representations allow the recipient to use the most convenient representation of location. There MAY be more than one PIDF-LO in the same SIP message, so long as each is in a separate message body part. Each location body part MAY point at different positions on the earth. The meaning of such a construction is not defined, and may cause confusion at the recipient. I would prefer that this is separated into sender and receiver requirements, and put into the appropriate subsections within section 5. Need to sort out the RFC 2119 language when doing so. |  |
| 139 | 5.1 | Editorial | Keith Drage | - 5.1, 1st para: A UAC may send location because it was requested to, to facilitate location-based routing, or spontaneously (i.e. a purpose not defined in this document but known to the UAC). A UAC MAY receive location from the UAS spontaneously. Change 1st "may" to "can". |  |
| 140 | 5.1 | NIT | Shida Schubert | 14. Section 5.1, 2nd paragraph > The terminology for Geolocation values seem to be inconsistent through out the document. It would be nice to have consistency for the type of URI used in Geolocation value. May be by-value URI and by-reference URI? |  |
| 141 | 5.1 | Editorial | Henning Schulzrinne | - The use of the option tag is confusing and should be described in the UAC/UAS sections, once. |  |
| 142 | 5.1 | NIT | Shida Schubert | 15. Section 5.1, 2nd last paragraph > "new Warning code" sounds like warning code yet to be defined. If you mean the warning code defined in the draft, it's better to be explicit about it. |  |
| 143 | 5.1 | Editorial | Henning Schulzrinne | - The text focuses on UAC-supplied information and only mentions a request mechanism (Require/Supported) in passing. This should be made much clearer. What exactly should happen if the UAC inserts a Require, but the UAS doesn't want to provide its location? Does it need to fail the call? This seems inappropriate. Can a UAS include location information without Supported, e.g., for the benefit of proxies? |  |
| 143A | 5.1 | Technical | Hannes Tschoenig | Section 5.1: " There MAY be future work defining additional error information, say in an XML body, indicating exactly what the error was, if any of the new Warning codes are ambiguous. " Delete this sentence -- RFC 2119 language not appropriate. |  |
| 144 | 5.1 | Editorial | Keith Drage | - 5.1, 7th para: There MAY be future work defining additional error information, say in an XML body, indicating exactly what the error was, if any of the new Warning codes are ambiguous. Inappropriate usage of RFC 2119 "MAY". Rewrite: "Additional error information indicating more specifically what the error is are outside the scope of this document, but could form the basis of future work" - if we need to say anything at all - it would be perfectly reasonable to delete this paragraph entirely. |  |
| 144A | 5.1 | Technical | Hannes Tschoenig | Section 5.1: " This is a seeking or pull model scenario, which is not defined here, and left for future study. " This document defines the ability to carry location information in a SUBSCRIBE/NOTIFY exchange. Let's assume it wouldn't be defined in this document then it would be available already via the rest of the presence work |  |
| 145 | 5.2 | Editorial | Henning Schulzrinne | - The use of the option tag is confusing and should be described in the UAC/UAS sections, once. |  |
| 146 | 5.2 | NIT | Shida Schubert | 16. Section 5.2, 2nd paragraph, 1st sentence. > "to" is missing OLD: A Require header with the geolocation option tag indicates the UAC is requiring the UAS understand this extension or else send an error response. NEW: A Require header with the geolocation option tag indicates the UAC is requiring the UAS to understand this extension or else send an error response. |  |
| 147 | 5.2 | Technical | Shida Schubert | 6. Section 5.2, 3rd paragraph, 1st sentence. > Do we want UAS to return an error even when location information is irrelevant to the dialog in question? Imagine a scenario where Alice's phone is set to always convey location info to her buddies even when the call has nothing to do with location. With the current spec simple call which has no business with location would fail if for example Alice's presence server is down.. Again there is no MUST or SHOULD on this paragraph so may be it's OK. |  |
| 148 | 5.2 | Technical | Shida Schubert | 6. Section 5.2, 3rd paragraph, 1st sentence. > Do we want UAS to return an error even when location information is irrelevant to the dialog in question? Imagine a scenario where Alice's phone is set to always convey location info to her buddies even when the call has nothing to do with location. With the current spec simple call which has no business with location would fail if for example Alice's presence server is down.. Again there is no MUST or SHOULD on this paragraph so may be it's OK. |  |
| 149 | 5.2 | Technical | Shida Schubert | 4. Section 5.2, 5th paragraph, 2nd sentence. > "but all MUST point at the same position on the earth." > This is not something one can enforce, I don't think UA generally has facility to translate geo<->civic. MUST strength seems a bit too strong here. May be SHOULD? |  |
| 150 | 5.2 | Editorial | Henning Schulzrinne | - The text focuses on UAC-supplied information and only mentions a request mechanism (Require/Supported) in passing. This should be made much clearer. What exactly should happen if the UAC inserts a Require, but the UAS doesn't want to provide its location? Does it need to fail the call? This seems inappropriate. Can a UAS include location information without Supported, e.g., for the benefit of proxies? |  |
| 151 | 5.2 | Technical | Henning Schulzrinne | - Normally, MIME body parts are meant to be self-describing. What happens if somebody simply includes a location information without any Geolocation header field? MUST this be ignored? (I can't look it up at 35,000 feet, but isn't there a content-disposition that should be used here?) |  |
| 152 | 5.2 | Technical | Henning Schulzrinne | - Is it possible for cid and inserted-by=server to occur? I assume this means that it was inserted by a B2BUA? What is the receiver supposed to do with this information? How does this relate to the PIDF-LO provided-by information? |  |
| 153 | 5.2 | Technical | Joszef Varga | (last para) The UAS behavior for sending location is the same as the UAC as above. - Some text for multi header processing? E.g. info provided by a proxy has higner (lower?: operators may not like to have a responsibility when UA inserted a location info, that can be "better") priority |  |
| 153A | 5.3 | Technical | Hannes Tschoenig | * Proxy Usage When a proxy adds a reference to a PIDF-LO then a number of issues remain unclear (as discussed throughout the Geopriv work), namely: - What does the PIDF-LO contain that is created by the SIP proxy? * How does the proxy know that location information has to be added? - There is a privacy problem if the SIP proxy acts without explicit indication that location information is added. |  |
| 154 | 5.3 | Editorial | Keith Drage | - 5.3, 1st para: [RFC3261] states message bodies cannot be added by proxies. However, a proxy may add a header to a message. This implies that a proxy MAY add a geolocation header with location-by-reference, but not location-by-value. Change 1st "may" to "can". The "MAY" is inappropriate here following the word "implies". We have to have certainties about RFC 2119 language. |  |
| 155 | 5.3 | Editorial | Keith Drage | - 5.3, 3rd para: More than one Geolocation header or header value in a message is permitted. The meaning of such a construction is not defined, and may cause confusion at the recipient. change "may" to "can". |  |
| 156 | 5.3 | Technical | Joszef Varga | (4th para) Proxies that perform location-based routing may need to consult external databases or systems to determine the route. Transmission of the location information (which SHOULD NOT reveal identity, even if the proxy knows the identity) is governed by the 'retransmission-allowed' and 'routing-query-allowed': Modify to: Proxies that perform location-based routing may need to consult external databases or systems to determine the route. Transmission of the location information (which SHOULD NOT reveal identity, even if the proxy knows the identity) is governed by the appearance of the 'retransmission-allowed' and 'routing-query-allowed' values in the 'usage rules' element of ...: |  |
| 157 | 5.3 | Editorial | Keith Drage | - 5.3 4th para: Proxies that perform location-based routing may need to consult external databases or systems to determine the route. Transmission of the location information (which SHOULD NOT reveal identity, even if the proxy knows the identity) is governed by the 'retransmission-allowed' and 'routing-query-allowed': change "may" to "can" |  |
| 158 | 5.3 | NIT | Shida Schubert | 17. Section 5.3, last paragraph. > "retransmission-allowed" and "routing-query-allowed" is an element in the PIDF-LO but it's only mentioned in the beginning of the draft. It may be nice to mention it again here. |  |
| 159 | 5.3 | Technical | Henning Schulzrinne | - Normally, MIME body parts are meant to be self-describing. What happens if somebody simply includes a location information without any Geolocation header field? MUST this be ignored? (I can't look it up at 35,000 feet, but isn't there a content-disposition that should be used here?) |  |
| 160 | 5.3 | Technical | Henning Schulzrinne | - Is it possible for cid and inserted-by=server to occur? I assume this means that it was inserted by a B2BUA? What is the receiver supposed to do with this information? How does this relate to the PIDF-LO provided-by information? |  |
| 161 | 5.3 | Technical | Henning Schulzrinne | - The Geolocation header field seems to cause retargeting. Wouldn't it be appropriate to indicate whether this causes the insertion of a History header field, say? |  |
| 162 | 5.3 | Technical | Henning Schulzrinne | - What should happen if a proxy tried to route on a location, failed for some reason ("bad" URL, access denied, ....), but then routed to some default location? Is that ok? |  |
| 163 | 5.3 | Technical | Richard Barnes | (4th para) Section 5.3, paragraph starting "Proxies that perform..." Need some text before the colon at the end of the paragraph, e.g., "fields in a PIDF-LO document". With regard to the table, would it be simpler just to say that the routing-query-allowed attribute supercedes the retransmission-allowed element? |  |
| 164 | 5.3.1 | Technical | Rishu Jain | 1) Multiple Locations: - Section 5.3.1 A server/proxy can add gelocation value to the existing geolocation header provided by UAC. In that case the messsage would look like this: Geolocation: <cid:alice123@atlanta.example.com>; inserted-by=endpoint, <sips:3sdefrhy2jj7@lis.atlanta.example.com>; inserted-by=server; =>How the location recipient will decide which location is to be referred to? Either the one provided in the Sip message (CID URI) or he has to subscribe to the location provided in SIPS URI? |  |
| 165 | 5.3.1 | Technical | Rishu Jain | 5) "message-routed-on-this-uri" tells the downstream entity that is was routed based on location once. In section 5.3.1 it is mentioned that in case multiple times if routing is done based on location still only one latest entry of "message-routed-on-this-uri" is sufficient. Why downstrem entity need to know that only one routing has happened (if at all it needs to know)? Why it need not find out multiple location based routing was done? |  |
| 166 | 5.3.1 | Technical | Henning Schulzrinne | - Removing the message-routed-by-this-uri seems unfounded if multiple routing operations take place. What makes the last one special? |  |
| 167 | 5.3.2 | Technical | Shida Schubert | 7. Section 5.3.2 > Proxy sending error when there is a location specific problem seems a bit problematic to me. I think you need to be clear about when this is appropriate. (malformed XML, can't decipher etc.) I think we want to prevent a case where proxy can't process the location information properly but UAS can. The only time proxy should return an error is if it is certain the location specific problem it sees is also problematic to the UAS. Furthermore, we don't want proxy not even doing location based routing to hinder a call just because it sees problem with location. |  |
| 167A | 6 | Technical | Hannes Tschoenig | Emergency case: I believe that all occurrences of emergency services should be deleted from the document since this document is generic in its nature. Let the Phone BCP deal with the emergency issue since it has todo so anyway |  |
| 168 | 6 | Technical | Richard Barnes | Security / Privacy Concerns: 1. In the first full paragraph of Section says that if an emergency call using TLS fails to complete, it MUST be retried without TLS. I don't think it's the place of the spec to mandate UA behavior in this case; it should be up to the user to define their urgency/privacy trade-off. |  |
| 169 | 6 | Technical | Richard Barnes | Security / Privacy Concerns: 2. In general most of section 6 is either out of scope or true in all contexts (not just emergency services). The only exceptions I note are the explanatory text at the beginning and the requirement that S/MIME MUST NOT be used in emergency cases. |  |
| 170 | 6 | Technical | Richard Barnes | Security / Privacy Concerns: 3. Saying that S/MIME MUST NOT be used for emergency calls rules out that security mechanism when UA itself has queried a LoST server and contacts a PSAP directly. Is this intentional? |  |
| 171 | 6 | Technical | Richard Barnes | Security / Privacy Concerns: 4. In Section 6, in the second to the last paragraph, it basically says for an emergency call certain policy flags "SHOULD" be set to "yes" and that if those flags are set to "no" or are not present then they may be ignored. A GEOPRIV "using protocol" shouldn't dictate that certain policy flags be ignored for "emergency calls" -- especially since emergency call marking is still poorly specified. |  |
| 172 | 6 | Technical | Richard Barnes | Security / Privacy Concerns: 5. On the other hand, if you're going to allow certain policy flags to be ignored, then shouldn't setting them to "yes" be a "MUST"? When would an entity legitimately set the policy flags to "no" in a situation where intermediate entities are allowed to ignore "no" values for these flags? |  |
| 173 | 6 | Editorial | Richard Barnes | (1st para, bullet 2) Section 6, bullet number 2 Remove "s/he", reword. |  |
| 174 | 6 | Editorial | Keith Drage | - 6, 3rd para: Recognizing which calls are emergency calls is beyond the scope of this document. When an emergency call is placed, location is normally provided by the UAC. Since emergency calls must be routed based on location (and indeed, in some jurisdictions, there may be several steps to such routing), the location must be visible to proxies along the path. Thus S/MIME protection of location MUST NOT be used. TLS protection of location SHOULD be used, however, if establishment of the TLS connection fails, the call set-up operation, including location conveyance, MUST be retried without TLS. Firstly emergency calls must be routed on location in some countries only. Secondly avoid pseudo RFC 2119 language. Suggest "must" --> "can". Change "may be" to "can be". |  |
| 175 | 6 | Editorial | Joszef Varga | (4th para) The entity inserting the geolocation header MUST specify the "inserted-by" parameter, with values of "endpoint" or "server" as appropriate. - "geolocation"  "Geolocation" |  |
| 176 | 6 | Editorial | Joszef Varga | (5th para) Both the "retransmission-allowed" and "routing-query-allowed" SHOULD be set to "yes". Querying for routing may be performed by proxies providing a routing service for emergency calls even if retransmission-allowed or routing-query-allowed is set to "no" or is not present. Proxies routing on the location MUST set the "message-routed-on-this-uri" parameter. - insert "elements" before "SHOULD". (or sg) |  |
| 177 | 6 | Editorial | Keith Drage | - 6, 5th para: Both the "retransmission-allowed" and "routing-query-allowed" SHOULD be set to "yes". Querying for routing may be performed by proxies providing a routing service for emergency calls even if retransmission-allowed or routing-query-allowed is set to "no" or is not present. Proxies routing on the location MUST set the "message-routed-on-this-uri" parameter. change "may be" to "can be". |  |
| 178 | 6 | Technical | Richard Barnes | (6th para) Section 6, paragraph starting "While many jurisdictions..." This should say that many jurisdictions "require" a user to reveal a location. It seems out of scope to mandate the configurability of positioning mechanisms. |  |
| 179 | 7 | Technical | Richard Barnes | (1st para) Section 7, paragraph starting "Transmitting location information..." Remove the word "Transmitting". Remove the word "tracking" from "eavesdropping, tracking, and alteration". Replace "any protocol wishing to be considered a GEOPRIV \"using protocol\"" with "a GEOPRIV using protocol". Changes "transport protocol meetings" to "transport protocol meets". I would remove the quotes from RFC 3693, and just cite the requirement numbers. |  |
| 179A | 7 | Technical | Hannes Tschoenig | * Section 7. The document refers to 'using protocols' for both the UAC <-> UAS and to the dereferencing protocol. I assume that Section 7 only addresses the UAC <-> UAS communication. Since I am in favor of not treating the derefencing protocol a 'using protocol' I am fine with it. Actually, I would prefer to move Section 7 into the appendix since this is just such an artifical story that it makes me nervous. It is not your fault; it is just the way how RFC 3693 was written. |  |
| 180 | 8 | Editorial | Keith Drage | - 8, 1st para: Conveyance of physical location of a UAC raises privacy concerns, and depending on use, there may be authentication and integrity concerns. This document calls for conveyance to normally be accomplished through secure mechanisms (like S/MIME or TLS). In cases where a session set-up is routed based on the location of the UAC initiating the session or SIP MESSAGE, securing the by-value location with an end-to-end mechanism such as S/MIME is problematic, because one or more proxies on the path need the ability to read the information to route the message appropriately. Securing the location hop-by-hop, using TLS, protects the message from eavesdropping and modification, but exposes the information to all proxies on the path as well as the endpoint. In most cases, the UAC does not know the identity of the proxy or proxies providing location-based routing services, so that end to middle solutions may not be appropriate either. change 1st "may" to "can". Change "end to middle solutions may not be appropriate either" to "end-to-middle solutions can also be inappropriate". |  |
| 181 | 8 | Technical | Richard Barnes | {2nd para} Section 8, paragraph starting "When the UAC is the source ..." Change the beginning of the first sentence to "When location is inserted by a UAC". What, exactly is being RECOMMENDED here? Should that clause be struck? |  |
| 181A | 8 | Technical | Hannes Tschoenig | * The security consideration section isn't particularly good. If I have time I rewrite it. |  |
| 181B | 8 | Technical | Hannes Tschoenig | The security consideration is far too short for this sensitive topic. |  |
| 182 | 9.4 | NIT | Martin Thomson | Editorial, Minor: 3.4 & 9.4: Warnings refer to unsupported "schema". This should be "scheme" [RFC3986]. |  |